Solar Audio Amplifier

DAY1

DAY2

## June 16

Built and simulated a silicon diode in PSpice.

Performed a DC voltage sweep and obtained the diode I-V characteristic.

Observed exponential current growth with increasing forward bias.

This serves as a precursor to photovoltaic cell modeling, since the solar-cell equivalent circuit is fundamentally diode-based.

On removing the Resistance and adding a current source instead

DAY-3

Day 3 – PV Model Improvement

- Added series resistance (Rs) and shunt resistance (Rsh) to the single-diode PV model.

- Tested different Rs and Rsh values in PSpice.

- Learned that circuit topology affects simulation results significantly.

- Connected a load resistor (RL) and obtained a stable output voltage.

- Performed an irradiance (Iph) sweep from 0.1 A to 1.0 A.

- Observed output voltage increase from approximately 0.81 V to 0.96 V with increasing irradiance.

Circuit used

Irradiance ( Iph ) sweep

Conclusion:
The PV model responds correctly to changes in illumination. Higher irradiance produces higher output voltage, indicating increased power generation capability.